# EOL Test Fixture User Manual

## Purpose

This manual is the primary operator guide for the End-of-Line (EOL) test fixture. It explains how to prepare the station, start a run, understand the screen flow, respond to pass or fail conditions, and export results.

This document is written for operators and technicians. It focuses on normal use of the fixture and avoids firmware-level detail unless it directly affects operator actions.

## Related Documents

Use this file as the main entry point. Refer to the following supporting documents as needed:

Preflight Checklist

Screen Guide

Troubleshooting Guide

Test Procedure

Error Code Reference

## Scope

This user manual applies to the current fixture revision for EOL_TestFixture_Final.

Current revision notes:

Digital Output 1 and Digital Output 2 are temporary automatic pass items in the present revision.

The Buttons / LEDs stage requires operator interaction.

Failed runs can still be exported after the system reaches Fault_Screen.

## Station Requirements

Before starting a run, make sure the following are available:

Powered and assembled test fixture

STM32 fixture firmware loaded

ESP32 EOL test firmware loaded

Correct DUT or simulated I/O connection

Correct cable set and wiring

Windows PC with export software available

ST-LINK connection for result export

Operator present for the full run, especially the Buttons / LEDs stage

For a run-by-run readiness check, use Preflight Checklist.

## Normal Operating Sequence

The production path is the automatic test flow. On the start screen, select Begin AUTO.

Do not use Begin MANUAL for normal production testing unless engineering or maintenance has specifically instructed you to use it.

### High-Level Flow

| Screen or Stage | What Happens | Operator Action |
| --- | --- | --- |
| Start Screen | Entry point to automatic or manual modes | Select Begin AUTO for production testing |
| Comm / Reset | Communication, MCU reset, and fault check run automatically | Observe status and wait |
| Inputs | Digital and analog input checks run automatically | Observe progress and do not interrupt |
| Outputs | Digital outputs auto-pass in this revision; relay checks run automatically | Observe progress and do not interrupt |
| Buttons / LEDs | Button checks and LED visual confirmation occur | Press and release the highlighted button when prompted, then select LED's GOOD or LED's BAD |
| Test Complete | Run passed | Record the unit as passed and export the results |
| Fault Screen | Run failed | Record the failure information and export the results |

## Before Starting A Run

Power the fixture and confirm the display starts normally.

Verify the DUT and all required cables are connected correctly.

Confirm the ESP32 has the correct EOL test firmware loaded.

Confirm the export PC is available if the run will be archived.

Confirm the operator can remain at the station during the Buttons / LEDs stage.

Review Preflight Checklist if there is any doubt about station readiness.

## Running The Automatic Test

### Step 1 - Start The Test

On the start screen, press Begin AUTO.

This starts the production test flow through the automatic screens in this order:

Comm_Reset_Screen

Inputs_Screen

Outputs_Screen

Buttons_LEDs_Screen

Test_Complete_Screen or Fault_Screen

### Step 2 - Observe Automatic Stages

The following stages run automatically:

Communication check

MCU reset verification

Fault check verification

Digital inputs

Analog inputs

Outputs and relay validation

During these stages:

allow the fixture to complete each stage

watch for labels turning green as steps pass

do not disconnect the DUT or cables

do not restart the run unless the procedure specifically requires it

### Step 3 - Complete The Buttons / LEDs Stage

The Buttons / LEDs screen requires operator action.

When the active button label turns yellow:

Press the indicated button.

Release the same button within the allowed time.

Repeat for each prompted button.

After all button checks pass:

Observe the LED pattern.

Press LED's GOOD if the LED behavior is correct.

Press LED's BAD if the LED behavior is incorrect.

## Pass And Fail Handling

### If The Run Passes

If the fixture reaches Test_Complete_Screen:

Record the unit as passed.

Export the completed run.

Confirm the expected result file is created on the PC.

### If The Run Fails

If the fixture reaches Fault_Screen:

Record the displayed error code.

Record the failed test or step name.

Record the expected and actual values shown on the screen.

Do not bypass the failure or continue testing without correction.

Export the failed run according to normal process.

Use Troubleshooting Guide and Error Code Reference to determine the next action.

## Exporting Results

After a completed pass or fail:

Open the export software on the PC.

Use Extract Data from NUCLEO-U575ZI-Q.

Retrieve the completed run from the STM32 over ST-LINK USART1.

Confirm a CSV file is created and stored in the expected export location.

If export fails:

verify the run has reached a terminal pass or fail state

verify the correct serial port is selected

verify the ST-LINK connection is present

review Troubleshooting Guide

## Operator Rules

Use Begin AUTO for normal production testing.

Stay at the fixture during the full run.

Do not ignore communication or reset failures.

Do not mark a unit as passed unless the fixture reaches Test_Complete_Screen.

Do not remove a failed unit before required information has been recorded.

Do not use the manual screen for production unless directed by engineering or maintenance.

## When To Use The Supporting Docs

Use Screen Guide if you want a screen-by-screen explanation of what to expect.

Use Troubleshooting Guide if a run fails and you need action-oriented recovery steps.

Use Error Code Reference if you need the exact meaning of a fault code.

Use Test Procedure for the formal detailed procedure and acceptance criteria.

Use Preflight Checklist before a shift or before a new run when setup is uncertain.