# EOL Test Fixture Screen Guide

Rite-Hite EOL Test Fixture Documentation

Generated from current project markdown on April 16, 2026

This guide explains what the technician sees on the fixture touchscreen and in the Windows desktop app, and what action is required at each point.

Use this with EOL Test Fixture User Manual, End-of-Line Test Procedure, and EOL Test Fixture Troubleshooting Guide.

# Color Meaning

## Fixture Touchscreen

| Color | Meaning |
| --- | --- |
| Black | Pending or not yet completed |
| Yellow | Active item or operator-attention state |
| Green | Passed |
| Red | Failed |

## Desktop App One Click Progress Rail

| Color | Meaning |
| --- | --- |
| Red | Current active One Click stage |
| Green | Completed stage |
| Gray | Not started yet |

# Fixture Touchscreen Screens

# Start Screen

## Purpose

Entry point to the fixture.

## What You See

Begin AUTO

Begin MANUAL

## What You Do

Press Begin AUTO for the normal production test.

Press Begin MANUAL only for service, debugging, or engineering checks.

## What Happens Next

Begin AUTO enters Comm_Reset_Screen.

Begin MANUAL enters Test_GPIO_Screen.

# Comm_Reset_Screen

## Purpose

Runs the first three fixture-controlled stages:

Communication

MCU Reset

Fault Check

## What You See

The stage labels update as each sub-stage passes or fails.

The progress bar advances through the three sub-stages.

## What You Do

Observe only.

Do not disconnect the DUT, fixture USB, or programmer during this stage.

## Result

Pass: fixture advances to Inputs_Screen

Fail: fixture transitions to Fault_Screen

# Inputs_Screen

## Purpose

Runs:

Digital Inputs

Analog Inputs

## What You See

Individual input items turn green as they pass.

The progress bar first covers digital inputs, then analog inputs.

## What You Do

Observe only.

Do not interrupt the run.

## Result

Pass: fixture advances to Outputs_Screen

Fail: fixture transitions to Fault_Screen

# Outputs_Screen

## Purpose

Runs:

Digital Outputs

Relay Outputs

## What You See

Digital Outputs and Relay Outputs are shown as separate sections.

Relay items update as K1 and K2 ON/OFF checks run.

## Current Revision Note

Digital Output 1 and Digital Output 2 are temporary auto-pass report rows.

Relay verification is live and can still fail.

## What You Do

Observe only.

## Result

Pass: fixture advances to Buttons_LEDs_Screen

Fail: fixture transitions to Fault_Screen

# Buttons_LEDs_Screen

## Purpose

Runs the final automatic button sequence and the final manual LED decision.

## What You See

Button labels for USR_0, PNL_0, PNL_1, PNL_2, and PNL_3

LED image / animation area

LED's BAD

LED's GOOD

## Important Current Behavior

The fixture automatically drives the button sequence by toggling its output lines and checking the ESP32 replies.

The technician does not manually press the hardware button sequence during the normal automatic test.

The technician only makes the final visual LED decision.

## What You Do

1. Watch the button stage complete automatically.

2. When the LED review begins, look at the LED pattern on the fixture.

3. Press LED's GOOD if the LED behavior is correct.

4. Press LED's BAD if the LED behavior is incorrect.

## Result

LED's GOOD: fixture advances to Test_Complete_Screen

LED's BAD: fixture transitions to Fault_Screen

# Fault_Screen

## Purpose

Final fail screen for the current run.

## What You See

Hex error code

Failed test or step name

Expected value or condition

Actual value or condition

## What You Do

1. Record the error information required by your process.

2. Use End-of-Line Error Code Reference and EOL Test Fixture Troubleshooting Guide.

3. Export the failed run.

# Test_Complete_Screen

## Purpose

Final pass screen for the current run.

## What You Do

1. Record the board or unit as passed.

2. Export the run, or confirm that One Click exported it automatically.

# Test_GPIO_Screen

## Purpose

Manual service and debug screen reached from Begin MANUAL.

## What You Can Control

DI1 through DI12 outputs

AUX0 through AUX3 button outputs

USR, MCU, and FAULT control outputs

DACA0, DACA1, and DACEN

DAC output slider

## What You Can Read

K1NO, K1NC, K2NO, K2NC, DO1, and DO2 input readback

## What You Do

Use only for maintenance, debug, or engineering-directed verification.

# Desktop App Screens

# File Explorer

## Purpose

Shows exported CSV files and lets the technician manage the export folder.

## What You See

File list of exported CSVs

Search box

Outcome filter

Buttons to refresh, open the folder, change the folder, or reset to default

## What You Do

Confirm the expected CSV exists after export.

Open or sort historical exports as needed.

# Device Tools

## Purpose

Provides the manual PC-side operations.

## What You See

Connection Readiness card

Last Successful Operation card

Extract Data from Test Fixture

Upload Test Firmware

Upload Production Firmware

## What You Do

Use Extract Data from Test Fixture after a normal touchscreen run to save the latest pass or fail report as a CSV.

Use Upload Test Firmware to load the fixed Rite-Hite Connect test firmware to the module.

Do not expect Upload Production Firmware to work yet; it currently shows a Coming Soon message.

# One Click Test

## Purpose

Runs the full PC-assisted workflow in one action.

## What You See

A connection-readiness card for the fixture and programmer

A large Start One Click Test button

A live progress rail:
  1. Upload Firmware
  2. Comms
  3. Digital Inputs
  4. Analog Inputs
  5. Digital Outputs
  6. Relay Outputs
  7. Buttons
  8. LEDs
  9. Extract Data

## What You Do

1. Confirm the fixture and programmer are detected.

2. Press Start One Click Test.

3. Watch the Activity Log and progress rail.

4. When the app indicates technician action is required, go to the fixture and press LED's GOOD or LED's BAD.

5. Confirm the CSV is saved at the end.

## How It Behaves

The progress rail follows the real STM32 fixture phase, not a fake timer.

The current phase is red.

The previous phase turns green when the next phase starts.

Manual extract and manual test-firmware upload are locked while One Click is running.

# Activity Log

## Purpose

Shows the detailed live log for uploads, One Click progress, prompts, and exports.

## What You Use It For

Confirm what step is active

See upload progress

See export completion

See One Click prompts such as the LED visual confirmation request

# Settings

## Purpose

Stores local app preferences.

## Available Settings

Language

Font size

Confirm before extract

Auto refresh on tab switch

Reduced motion

Dark mode setting

Custom exports folder path

## Language Support

The app includes:

English

Spanish

Hmong