# EOL Test Fixture & Export Software Architecture & Design

Rite-Hite EOL Test Fixture Documentation

This file contains the firmware and software architecture of both the EOL Test Fixture and EOL Export Software for project-specific maintenance.

For example, use it when you need to answer questions such as:

Which file owns a given part of the EOL Test Fixture behavior

Where to change protocol messages, fault codes, or timeouts

How the TouchGFX UI is tied into the test engines

Which EOL Export Software file owns export, settings, or One Click Test behavior

# File Ownership Map

## STM32 Core Firmware (Fixture)

| File | Responsibility |
| --- | --- |
| Core/Inc/eol_test_protocol.h | Shared protocol strings, timing constants, and EOL_ERR_* fault codes |
| Core/Src/comm_test.c | Communication, reset, and fault-check state machine |
| Core/Src/di_test.c | Digital-input stage |
| Core/Src/ai_test.c | Analog-input stage |
| Core/Src/output_test.c | Output and relay stage |
| Core/Src/button_led_test.c | Automatic button sequence and final LED decision wait |
| Core/Src/eol_report.c | Run report rows and report state machine |
| Core/Src/export_uart.c | EXPORT_STATUS, EXPORT_RUN, and START_AUTO services |
| Core/Src/auto_test.c | Auto-test start flag and phase tracking |
| Core/Src/gpio_config.c | Fixture GPIO ownership and DAC / mux support |
| Core/Src/uart_config.c | UART4 transport to the ESP32 |

## TouchGFX Layer (Display)

| File | Responsibility |
| --- | --- |
| TouchGFX/gui/src/model/Model.cpp | Tick loop, engine start calls, auto-test phase updates |
| TouchGFX/gui/src/common/FrontendApplication.cpp | Shared fault info and forced screen transition helpers |
| TouchGFX/gui/src/*Presenter.cpp | Per-screen activation and interaction wiring |
| TouchGFX/gui/src/containers/*.cpp | UI response to test progress, pass, and fail |
| TouchGFX/gui/src/buttons_leds_screen_screen/Buttons_LEDs_ScreenView.cpp | LED's GOOD and LED's BAD actions |

## Export Software (Desktop App)

| File | Responsibility |
| --- | --- |
| Export_Software/main.py | UI, One Click Test worker, progress rail, activity log, page layout |
| Export_Software/serial_client.py | Fixture serial protocol client |
| Export_Software/esp32_uploader.py | Rite-Hite Connect test-firmware upload path |
| Export_Software/storage.py | .csv writing, settings, last-successful-operation persistence |
| Export_Software/translations.py | English, Spanish, and Hmong strings |
| Export_Software/theme.py | Visual styling |

## ESP32 Test Firmware (DUT)

| File | Responsibility |
| --- | --- |
| ArduinoIDE/EOL_RiteHite_Final/EOL_RiteHite_Final.ino | Protocol token handling, expander control, analog reads, relay and button acknowledgments |

# Common Change Map

## If You Need To Change A Fault Code

Update:

Core/Inc/eol_test_protocol.h

The stage engine that raises the code

Any TouchGFX fault-text mapping for that stage

Docs/error_code_reference.md

Docs/troubleshooting_guide.md if operator action changes

## If You Need To Change The One Click Test Progress Rail

Update:

STM32 auto-test phase ownership in auto_test.c

Phase transitions in Model.cpp

Status parsing in serial_client.py

Progress mapping in main.py

Docs/screen_guide.md and Docs/user_manual.md

## If You Need To Change Export Behavior

Update:

eol_report.c if report content changes

export_uart.c if protocol framing changes

serial_client.py if desktop parsing changes

storage.py if CSV structure changes

Docs covering export format and operator workflow

## If You Need To Change The Button / LED Stage

Update:

button_led_test.c

Model.cpp

Buttons_LEDs_ScreenPresenter.cpp

Buttons_LEDs_ScreenView.cpp

export_uart.c if visual-wait semantics change

One Click docs if the technician action changes

# Current Protocol Quick Reference

## STM32 (Fixture) To ESP32 (DUT) Core Tokens

PING

READY

DIxTEST

AIx_LOW_REQ

AIx_MID_REQ

AIx_HIGH_REQ

K1_ON

K1_OFF

K2_ON

K2_OFF

BTN_BEGIN

LED_BEGIN

LED_ROTATE_250

LED_STOP_ALL_ON

## Expected ESP32 (DUT) Replies

PONG

OK

DIxOK

DIxFAIL

OK_AI...:<volts>

OK_K1_ON, OK_K1_OFF, OK_K2_ON, OK_K2_OFF

OK_BTN_...

negative button acknowledgments such as BTN_USR0_PRESS_HIGH

OK_LED_BEGIN

OK_LED_ROTATE_250

OK_LED_STOP_ALL_ON

## Export Software To Test Fixture Tokens

START_AUTO

EXPORT_STATUS

EXPORT_RUN|<sequence>

## Test Fixture To Export Software App Replies

AUTO_STARTED

AUTO_ERROR|ALREADY_RUNNING

STATUS|...

EXPORT_BEGIN|...

ROW|...

EXPORT_END|...

EXPORT_ERROR|...

# Current Test Ownership Notes

## Communication / Reset / Fault Check

Single state machine in comm_test.c

Fault family: 0x01xx, 0x02xx, 0x03xx

## Digital Inputs

12 channels

Expected result text: High then Low

Fault family: 0x04xx

## Analog Inputs

4 channels

3 levels per channel

Fault family: 0x05xx

## Outputs

DO1 and DO2 are temporary auto-pass rows

Relay verification is live

Fault family: 0x06xx

## Buttons / LEDs

Button actuation is automatic

LED visual decision is manual

Fault family: 0x07xx

# TouchGFX Integration Rules

The current EOL Test Fixture project uses the TouchGFX model as the coordinator between the non-blocking test engines and the screen presenters.

Practical rules:

Start engines from presenters through model methods

Surface per-stage progress back through the model or container callbacks

Keep the report and fault ownership in the firmware engine, not in the container

Use Fault_Screen only after fault data has been fully populated

Keep One Click phase reporting aligned with actual engine state, not UI guesses

# EOL Export Software Integration Rules

## Serial Client

serial_client.py is the source of truth for the desktop app's fixture protocol parsing.

If the export UART format changes, update this file first.

## Storage

storage.py owns:

exports directory selection

timestamp-based .csv naming

.csv column names

settings persistence

last successful operation persistence

## One Click Test

main.py owns:

The One Click Test worker

The app-side phase mapping

Live Activity Log updates

UI lockouts during One Click Test

Note: The EOL Export Software must continue to treat WAITING_VISUAL as a real stop point that requires a technician action on the fixture.

# Code Maintenance

## Safe Change Checklist

Before committing a behavior change, ask:

Does it change a protocol token?

Does it change a timing constant?

Does it change what the operator must do?

Does it change what One Click Test displays or blocks?

Does it change what gets written to CSV?

Does it change a fault code or fault text?

If the answer is yes to any of these, update the matching docs in Docs.

## Current Known Constraints

Production firmware upload is not implemented in the EOL Test Fixture Software.

Digital Output 1 and Digital Output 2 are not yet real validated output tests in the fixture flow.

Why? PCB board was in development and did not have correct outputs at time of design.

The exported .csv contains human-readable test rows but not a raw fault-code column.

## Recommended Validation After Changes

### For STM32 (EOL Test Fixture Firmware) Changes

Build the TouchGFX target

Confirm the expected screen transitions still occur

Confirm Fault_Screen text still matches the new behavior

Confirm export still works for pass and fail

### For ESP32 (DUT Test Firmware) Protocol Changes

Confirm the token strings still match the STM32 expectations exactly

Confirm negative acknowledgments still map cleanly to fault codes

Confirm analog reply formatting still matches STM32 parsing

### For EOL Export Software Changes

Run py -m py_compile on the app modules

Verify manual extract

Verify test-firmware upload

Verify One Click start, live progress, LED wait, and final export

## Documentation Rules

- Do not leave docs describing old behavior when the code has moved on.

- For this project, documentation is required whenever any of these change:

Fixture workflow

Operator actions

Export protocol

One Click Test behavior

Active fault codes

Hardware ownership