# EOL Test Fixture Preflight Checklist

Rite-Hite EOL Test Fixture Documentation

Use this checklist at shift start, after maintenance, after a firmware change, after a wiring change, or any time station readiness is uncertain.

For detailed operating instructions, see the EOL Test Fixture User Manual.

# Station Record

| Record Field | Entry |
| --- | --- |
| Date |  |
| Operator / Technician |  |
| Fixture ID |  |
| DUT or Sample Board ID |  |
| PC Name |  |
| Notes |  |

# Fixture Readiness

[ ] Fixture powers on normally with the E-STOP button and power plug.

[ ] Touchscreen is visible and responds to touch.

[ ] Begin Automatic Test and Begin Manual Test are visible on the start screen.

[ ] No loose wiring, damaged connectors, or visible hardware damage are present.

[ ] The Rite-Hite Connect Module (DUT) for this run is installed correctly.

# Firmware And Software Readiness

[ ] The EOL Test Fixture Firmware is loaded and the fixture reaches the normal start screen.

[ ] The Rite-Hite Connect Module has the correct test firmware loaded, or the programmer is available to load it from the EOL Export Software.

[ ] The EOL Export Software export opens normally and is ready to test.

[ ] The EOL Export Software language, folder path, and settings are acceptable for the shift.

[ ] The EOL Export Software data export folder is in the desired location, reachable, and writable.

# Connection Readiness

## Test Fixture / ST-LINK Path

[ ] EXPORT: The USB-C labeled “EXPORT” on the EOL Test Fixture is connected to the PC.

[ ] The EOL Export Software can detect the fixture serial port (USB-C), or the technician knows which ST-LINK COM port to select manually.

## Programmer Path

[ ] UPLOAD: The USB-C labeled “UPLOAD” on the EOL Test Fixture for Rite-Hite Connect Module programming is connected to the PC.

[ ] The EOL Export Software shows the programmer as detected before attempting Upload Test Firmware or starting the One Click Test.

# Workflow Selection

Choose the workflow that will be used for this station cycle.

[ ] Standard Fixture Workflow: Start the run from the touchscreen and export with the Device Tools section in the EOL Export Software after pass or fail.

[ ] One Click Workflow: Use the EOL Export Software by clicking Start One Click Test in the One Click Test section to upload DUT firmware, start the fixture automatically, track progress, and export the data into a .csv automatically.

# Operator Reminders

[ ] Begin Automatic Test is the normal production path on the fixture.

[ ] Begin Manual Test is the service GPIO screen and is not part of normal production testing. It is used for Engineering troubleshooting of the EOL Test Fixture.

[ ] The EOL Test Fixture automatically drives the button sequence during the Buttons test stage.

[ ] The technician must make the LED visual decision by pressing LED's GOOD or LED's BAD on the fixture touchscreen.

[ ] Runs can be exported after the display reaches the successful test screen or the failure screen.

# Current Revision Notes

[ ] Digital Output 1 and Digital Output 2 are temporary auto-pass report rows in the current revision of the DUT.

[ ] The EOL Export Software app production firmware upload card is present, but the production upload action is not available yet.

[ ] One Click Test locks out manual extract and manual test-firmware upload while the One Click run is active. It is designed to be completely hands-off, except for technician LED decisions.

# Ready To Start

[ ] All checks above are complete.

[ ] The Rite Hite Connect Module (DUT) is connected correctly.

[ ] The selected workflow is understood by the operator.

[ ] The station is ready for a production run.

# If Anything Is Not Ready

Stop before starting the run.

Correct the setup issue first.

For persistent issues, use the EOL Test Fixture Troubleshooting Guide.

Escalate to maintenance or engineering if the issue is outside normal operator recovery.