# EOL Test Fixture Screen Guide

Rite-Hite EOL Test Fixture Documentation

This guide explains what the technician sees on the EOL Test Fixture touchscreen and in the EOL Export Software app, including what action is required at each point.

Use this with EOL Test Fixture User Manual, EOL Test Fixture Test Procedure, and EOL Test Fixture Troubleshooting Guide.

# Color Meaning

## Fixture Touchscreen

| Color | Meaning |
| --- | --- |
| Black | Pending or Not Yet Reached |
| Yellow | Active Test Item or Operator-Attention State |
| Green | Passed |
| Red | Failed |

## Desktop App One Click Test Progress Rail

| Color | Meaning |
| --- | --- |
| Red | Currently Active One Click Test Stage |
| Green | Completed stage |
| Gray | Not Started Yet |

# EOL Test Fixture Screens

## Start Screen

### Purpose

Entry point to the fixture.

### What You See

Begin Automatic Test

Begin Manual Test

### What You Do

Press Begin Automatic Test for the normal production test.

Press Begin Manual Test only for service, debugging, or engineering checks.

### What Happens Next

Begin Automatic Test begins the production test and enters the Communication Test Screen.

Begin Manual Test enters the Manual Configuration Screen.

## Communication Test Screen

### Purpose

Runs the first three fixture-controlled stages:

Communication

MCU Reset

Fault Check

### What You See

The stage labels update as each sub-stage passes or fails.

The progress bar advances through the three sub-stages.

### What You Do

Observe only.

Do not disconnect the DUT, fixture USB, or programmer during this stage.

### Result

Pass: Fixture advances to Input Test Screen

Fail: Fixture transitions to Fault Screen

## Input Test Screen

### Purpose

Runs tests for:

Digital Inputs

Analog Inputs

### What You See

Individual input items turn green as they pass.

The progress bar first covers digital inputs, then analog inputs.

### What You Do

Observe only.

Do not interrupt the run.

### Result

Pass: Fixture advances to Output Test Screen

Fail: Fixture transitions to Fault Screen

## Output Test Screen

### Purpose

Runs tests for:

Digital Outputs

Relay Outputs

### What You See

Digital Outputs and Relay Outputs are shown as separate sections.

Relay items update as K1 and K2 ON/OFF checks run.

### Current Revision Note

Digital Output 1 and Digital Output 2 are temporary auto-pass report rows.

Relay verification is live and can still fail.

### What You Do

Observe only.

Do not interrupt the run.

### Result

Pass: fixture advances to Button & LED Test Screen

Fail: fixture transitions to Fault Screen

## Button & LED Test Screen

### Purpose

Runs the final automatic button sequence and the final manual LED decision.

### What You See

Button labels for USR_0, PNL_0, PNL_1, PNL_2, and PNL_3

LED image / animation area

LED's BAD

LED's GOOD

### Important Current Behavior

The fixture automatically drives the button sequence by toggling its output lines and checking the ESP32 replies. Do not attempt it manually during the normal automatic test.

The technician only makes the final visual LED decision. The test will wait until the decision.

### What You Do

Watch the button stage complete automatically.

When the LED review begins, look at the LED pattern on the fixture.

Press LED's GOOD if the LED behavior is correctly matches the rotation shown on screen.

Press LED's BAD if the LED behavior is incorrectly matches the rotation shown on screen.

### Result

LED's GOOD: Fixture advances to Test Complete Screen

LED's BAD: Fixture transitions to Fault Screen

## Fault Screen

### Purpose

Final fail screen for the current run.

### What You See

Hex error code (i.e. 0x0xxx)

Failed test or step name

Expected value or condition

Actual value or condition

### What You Do

- Record the error information required by your process.

- Use EOL Test Fixture Error Code Reference and EOL Test Fixture Troubleshooting Guide.

- Export the failed run.

## Test Complete Screen

### Purpose

Final pass screen for the current run.

### What You Do

- Record the board or unit as passed.

- Export the run, or confirm that One Click exported it automatically.

## Manual Configuration Screen

### Purpose

Manual service and debug screen reached from Begin Manual Test for engineering purposes.

### What You Can Control

DI1 through DI12 outputs

AUX0 through AUX3 button outputs

USR, MCU, and Fault control outputs

DACA0, DACA1, and DACEN

DAC output slider

### What You Can Read

K1NO, K1NC, K2NO, K2NC, DO1, and DO2 input readback

### What You Do

Use only for maintenance, debug, or engineering-directed verification.

# EOL Export Software Screens

## File Explorer

### Purpose

Shows exported .csv files. Allows the technician to open them and manage the export folder.

### What You See

File list of exported data in .csv format

Search box

Outcome filter

Buttons to refresh, open the folder, change the folder, or reset to default

### What You Do

Confirm the expected .csv exists after export.

Open or sort historical exports as needed.

## Device Tools

### Purpose

Provides the manual PC-side operations.

### What You See

Connection Readiness card

Last Successful Operation card

Extract Data from Test Fixture

Upload Test Firmware

Upload Production Firmware

### What You Do

Use Extract Data from Test Fixture after a normal touchscreen run to save the latest pass or fail report as a .csv data format file.

Use Upload Test Firmware to load the fixed Rite-Hite Connect Module test firmware.

Do not expect Upload Production Firmware to work yet; A coming soon message is shown.

## One Click Test

### Purpose

Runs the full PC-assisted workflow in one action.

### What You See

A connection-readiness card for the fixture and programmer

A large Start One Click Test button

A live progress rail: 1. Upload Firmware 2. Comms 3. Digital Inputs 4. Analog Inputs 5. Digital Outputs 6. Relay Outputs 7. Buttons 8. LEDs 9. Extract Data

### What You Do

- Confirm the fixture and programmer are detected.

- Press Start One Click Test.

- Watch the Activity Log and progress rail.

- When the app indicates technician action is required, go to the fixture and press LED's GOOD or LED's BAD after evaluation.

- Confirm the .csv is saved at the end.

### How It Behaves

The progress rail follows the real EOL Test Fixture phase (test state), not a fake timer.

The current phase is red.

The previous phase turns green when the next phase starts.

Manual extract and manual test-firmware upload are locked while One Click Test is running.

## Activity Log

### Purpose

Shows the detailed live log for uploads, One Click Test progress, prompts, and exports.

### What You Use It For

Confirm what step is active

See upload progress

See export completion

See One Click Test prompts such as the LED visual confirmation request

## Settings

### Purpose

Stores local app preferences.

### Available Settings

Language

Confirm before extract

Auto refresh on tab switch

Reduced motion

Dark mode setting

The export folder path is managed from File Explorer and stored locally on the PC when it is changed.

### Language Support

The app includes:

English

Spanish

Hmong