# EOL Test Fixture Test Procedure

Rite-Hite EOL Test Fixture Documentation

This document defines the current production procedure for the Rite-Hite EOL Test Fixture.

It covers the following:

The Standard Fixture-Driven Workflow

The EOL Export Software One Click Test Workflow

Required Operator Actions

Pass/Fail Handling

.csv Export Handling

Use this with EOL Test Fixture User Manual, EOL Test Fixture Preflight Checklist, and EOL Test Fixture Error Code Reference.

# Scope

This procedure applies to the current fixture revision built around:

STM32U575 Fixture Firmware and TouchGFX UI (EOL Test Fixture)

ESP32-S3 Rite-Hite Connect Test Firmware

Windows PySide6 Export Application (EOL Export Software)

# Roles

## Technician / Operator

Connect the Rite Hite Connect Module (DUT) to the fixture

Start the run using the selected workflow (Automatic or Manual Test)

Make the final LED visual decision

Export or confirm the exported .csv

Record Pass/Fail disposition

## Engineer / Maintenance

Maintain EOL Test Fixture firmware and station wiring

Use the Manual Configuration Screen when required for debugging

# Preconditions

Before starting:

The EOL Test Fixture is powered and reaches the start screen

The DUT or harness is connected correctly

The EOL Export Software opens normally

The serial path is available for export

The programmer path is available if firmware upload is required

The operator is ready to make the final LED decision

# Test Sequence Overview

The fixture automatic flow runs in this order:

Communication

MCU Reset

Fault Check

Digital Inputs

Analog Inputs

Digital Outputs

Relay Outputs

Buttons

LED Visual Check

## Current Revision Note

Digital Output 1 and Digital Output 2 are temporary auto-pass rows.

The fixture automatically drives the button sequence. Do not press them manually.

The LED decision is still manual decision by the operator.

# Procedure A: Standard Fixture Workflow

Use this procedure when the run is started from the EOL Test Fixture touchscreen and exported afterward from Device Tools in the EOL Export Software.

## Step 1: Prepare The Station

- Complete the readiness checks in EOL Test Fixture Preflight Checklist.

Confirm the EOL Test Fixture start screen is visible.

Confirm the correct test firmware is available on the Rite-Hite Connect Module, or load it from the EOL Export Software before running.

## Step 2: Start The Automatic Fixture Run

- On the touchscreen, press Begin Automatic Test.

Confirm that the fixture enters Communication Test Screen.

## Step 3: Allow Automatic Stages To Complete

The fixture runs the following without operator input:

Communication

MCU Reset

Fault Check

Digital Inputs

Analog Inputs

Outputs

Buttons

During this time:

Do not disconnect cables

Do not power-cycle the station

Do not start a second test

## Step 4: Perform The LED Visual Decision

When the fixture reaches the LED visual review:

- Observe the LED pattern.

Press LED's GOOD if correct.

Press LED's BAD if incorrect.

## Step 5: Record The Fixture Result

If the run passes:

The fixture reaches Test Complete Screen.

If the run fails:

The fixture reaches Fault Screen.

Record the fault code, failed step, expected value, and actual value as required by local process.

## Step 6: Export The Run

- Open the EOL Export Software.

Go to Device Tools.

Press Extract Data from Test Fixture.

Confirm that one .csv is created for the run.

Verify that the file appears in File Explorer.

# Procedure B: One Click Workflow

Use this procedure when the EOL Export Software will upload the test firmware, start the run, track progress, and export the .csv automatically, without the need of the touchscreen on the EOL Test Fixture.

## Step 1: Confirm Readiness In The App

On the One Click Test page:

- Confirm the fixture is detected.

Confirm the programmer is detected.

Confirm the Start One Click Test button is enabled.

## Step 2: Start One Click

- Press Start One Click Test.

Confirm the Activity Log begins showing upload and startup messages.

## Step 3: Let One Click Test Run

The EOL Export Software performs the following:

- Uploads the Rite-Hite Connect Module test firmware

Waits for module boot

Sends Start Automatic Test to the fixture

Polls fixture status during the run

Advances the progress rail as the fixture changes stage

The progress rail follows:

- Upload Firmware

Comms

Digital Inputs

Analog Inputs

Digital Outputs

Relay Outputs

Buttons

LEDs

Extract Data

## Step 4: Make The LED Decision

When the Activity Log indicates technician action is required:

- Go to the fixture touchscreen.

Observe the LED pattern.

Press LED's GOOD or LED's BAD.

## Step 5: Confirm Automatic Export

At the end of a successful One Click Test flow:

- The EOL Export Software exports the completed run automatically.

The Activity Log reports completion.

The .csv appears in File Explorer.

If One Click Test ends in a EOL Test Fixture failure:

The app still exports the completed failed run once the fixture report reaches the ready state.

# Pass Criteria

The unit is a PASS only when all of the following are true:

Communication passes

MCU Reset passes

Fault Check passes

All digital input checks pass

All analog input checks pass

Output and relay checks pass under the current revision rules

The button sequence completes successfully

The technician presses LED's GOOD

The run reaches Test Complete Screen or a One Click Test pass result is reported

# Fail Criteria

The unit is a FAIL if any of the following occur:

Any fixture stage returns an error

A relay verification fails

A button or LED stage returns a protocol, timeout, or state-mismatch error

The technician presses LED's BAD

The fixture reaches Fault Screen

# Output And Reporting

## Fixture Report Behavior

The fixture builds a report row for each checked item during the run.

Completed runs have one of these report states:

READY: completed and not yet exported

EXPORTED: already exported once

## Data Output (.csv)

Each export creates one .csv file containing:

Date and Time

Test name

Expected Result

Actual result

Pass/Fail

## File Naming

.csv files are named using:

YYYY-MM-DD_HH-MM-SS_<outcome>.csv

# Operator Warnings

Do not bypass communication, reset, or fault-check failures.

Do not assume Digital Output 1 and Digital Output 2 are fully validated in the current revision; they are temporary auto-pass rows.

Do not walk away before the LED visual decision is made.

Do not attempt manual extract or manual firmware upload while One Click Test is running.

Do not expect the EOL Export Software production-firmware upload action to work yet.

# Records

Recommended run record fields:

Date and time

Operator name

Fixture ID

DUT or unit identifier

Pass or Fail result

Fault code if failed

.csv filename

# Related Documents

EOL Test Fixture User Manual

EOL Test Fixture Screen Guide

EOL Test Fixture Troubleshooting Guide

EOL Test Fixture Error Code Reference