# EOL Test Fixture Troubleshooting Guide

Rite-Hite EOL Test Fixture Documentation

This guide is written for technicians and operators who need a clear next action when the EOL Test Fixture or EOL Export Software does not behave as expected. Manual Test mode may help debug.

Use this with EOL Test Fixture Error Code Reference for exact code meanings.

# Immediate Actions After Any Failure

If the fixture reaches Fault Screen.

Record the fault code.

Record the failed step.

Record the expected and actual values if required by local process.

Export the failed run.

Correct the likely cause before starting another run.

If the EOL Export Software reports a failure during One Click Test:

- Read the latest Activity Log entry.

Determine whether the failure is in firmware upload, EOL Test Fixture start, live run status, LED wait, or export.

Correct that issue before restarting One Click Test.

# General Rules

Do not bypass communication, reset, or fault-check failures.

Do not rerun the same failing setup repeatedly without checking the station.

If the same failure appears on multiple boards, suspect the station before the DUT.

If the EOL Export Software and EOL Test Fixture disagree about the state of the run, trust the fixture screen first, then export and inspect the .csv.

# Quick Symptom Guide

| Symptom | Likely Cause | Required Action |
| --- | --- | --- |
| No fixture COM port appears | PC connection / ST-LINK | Check USB cable, ST-LINK connection, Device Manager, and correct fixture port |
| Programmer not detected | Programmer path | Check programmer cable, USB connection, and that only the intended programmer is attached |
| One Click Test will not start | App readiness | Confirm fixture port detected, programmer detected, and no other One Click run is active |
| One Click Test stops at LEDs | Normal manual hold point | Go to the fixture and press LED's GOOD or LED's BAD |
| Export says not ready | Run still active | Confirm the EOL Test Fixture has reached Test Complete Screen or Fault Screen |
| Export says already exported | Report state | Check File Explorer for the existing .csv |
| Repeated relay failures | Output hardware or readback | Check relay wiring, contact readback, and relay board behavior |
| Button or LED state mismatch | Button/LED interface | Check Q3 inputs, button wiring, and LED hardware |

# EOL Test Fixture Failures

## Communication Failures ( 0x0101 , 0x0102 )

Meaning:

The fixture did not receive the expected PONG

Or it received the wrong reply

First checks:

- Confirm the Rite-Hite Connect Module is powered.

Confirm the correct ESP32 test firmware is loaded onto the Rite-Hite Connect Module.

Check the STM32 to ESP32 serial wiring (Rite-Hite Connect Module to EOL Test Fixture).

Check for loose connectors.

Escalate if:

The same fault repeats after power and wiring are confirmed.

## MCU Reset Failures ( 0x0201 to 0x0203 )

Meaning:

The reset pulse path failed

Or the target did not come back correctly after reset

First checks:

- Check the reset wiring and hardware path.

Confirm the target reboots normally.

Confirm the target returns the expected reply after reset.

Escalate if:

Reset failures continue on known-good targets.

## Fault Check Failures ( 0x0301 to 0x0303 )

Meaning:

The fault pulse path or acknowledgment failed

First checks:

- Inspect the fault control path.

Verify wiring from the EOL Test Fixture to the target.

Confirm the target responds to the fault-check logic.

## Digital Input Failures ( 0x0401 to 0x0403 )

Meaning:

The target did not acknowledge the digital input test

Or it reported the wrong state transition

First checks:

- Check the digital input wiring.

Confirm the correct board or harness is attached.

Inspect the affected channel connection.

## Analog Input Failures ( 0x0501 to 0x0503 )

Meaning:

No analog reply

Invalid analog reply

Measured value outside tolerance

First checks:

- Check analog wiring and multiplexer path on the relay board inside the EOL Test Fixture.

Confirm the DAC and analog route are operating in the EOL Test Fixture.

Compare the expected and actual reading shown on the Fault Screen.

## Output And Relay Failures ( 0x0601 to 0x0603 )

Meaning:

Relay command acknowledgment failed

Or relay NO/NC readback did not match expectation

Important note:

In the current revision, Digital Output 1 and Digital Output 2 are temporary auto-pass rows.

Real failures in this stage are normally relay failures.

First checks:

- Check relay wiring.

Check K1 and K2 readback signals.

Confirm contact polarity is correct.

## Buttons And LED Failures ( 0x0701 to 0x0705 )

Meaning:

No reply, invalid reply, timeout, operator reject, or state mismatch during the Buttons / LEDs stage

Important note:

The fixture automatically drives the button sequence.

The operator only makes the final LED visual decision.

First checks:

- Check the button and LED interface wiring.

Check the Q3 input readings on the ESP32 side.

Confirm the LED pattern actually matches expected behavior before choosing LED's BAD.

If the code is 0x0705, inspect the measured button state path because the target saw the opposite state from what the EOL Test Fixture expected.

# EOL Export Software Problems

## EOL Test Fixture Not Detected

Symptoms:

Connection Readiness shows no serial ports

Extract Data from Test Fixture cannot start

One Click does not allow start

Checks:

- Reconnect the fixture USB-C cable labeled “EXPORT”.

Confirm the ST-LINK virtual COM port appears in Windows.

If multiple ST-LINK ports exist, choose the correct one manually when prompted.

## Programmer Not Detected

Symptoms:

Upload Test Firmware cannot start

One Click start is blocked

Checks:

- Reconnect the programmer USB-C cable labeled “UPLOAD”.

Confirm the intended programmer is attached.

Remove extra programmer devices if multiple similar ports are present.

## One Click Test Will Not Start

Checks:

- Confirm the fixture is detected (“EXPORT” cable is connected).

Confirm the programmer is detected (“UPLOAD” cable is connected).

Confirm no other One Click Test run is active.

Check the Activity Log for the most recent error and record it.

## One Click Test Appears Stuck

Most common causes:

Waiting for the module to boot after firmware upload

Waiting for the fixture to finish a stage

Waiting for the LED visual decision

Checks:

- Look at the active red stage in the One Click Test progress rail, record it.

Read the latest Activity Log entries.

If the software is waiting at LEDs, go to the EOL Test Fixture and make the LED decision.

If the software times out after 5 minutes, inspect the EOL Test Fixture screen and target the hardware manually with the touchscreen display.

## Manual Extract Is Blocked During One Click Test

This is expected behavior.

The EOL Export Software locks manual extract and manual test-firmware upload while One Click Test is active to prevent a corrupted or partial run flow.

# Export Problems

## Export Reports Not Ready

Meaning:

The fixture report is still running or waiting for the LED decision.

Action:

- Confirm the run has fully completed.

If the fixture is waiting for the LED decision, finish that step first.

## Export Reports Already Exported

Meaning:

That run has already been exported once.

Action:

- Check File Explorer for the existing .csv.

If another copy is required, export the file from the folder rather than expecting the fixture to regenerate the same run.

## Export Reports Bad Sequence

Meaning:

The requested report sequence does not match what the fixture holds.

Action:

- Refresh the software state.

Query the fixture again.

Retry export from the latest available run only.

## Activity Log And UI Issues

If the software UI behaves unexpectedly:

- Read the latest Activity Log message first.

If the window layout looks wrong, exit fullscreen or maximized mode with ESC key, then restore the desired view.

Restart the software if the UI no longer responds.

If the issue repeats, escalate with:

Screenshot of the software issue

Relevant Activity Log lines

Whether the issue occurred in Device Tools or One Click Test

# When To Escalate

Escalate to engineering or maintenance when:

The same failure repeats on multiple units

Communication, reset, or fault-check failures continue after basic checks

Relay readback does not match physical behavior

Button/LED mismatches continue on known-good hardware

The EOL Export Software repeatedly cannot detect the “UPLOAD” or “EXPORT” USB-C ports on a known-good PC

The station shows signs of wiring damage or unstable power

# Related Documents

EOL Test Fixture User Manual

EOL Test Fixture Screen Guide

EOL Test Fixture Test Procedure

EOL Test Fixture Error Code Reference