# EOL Test Fixture User Manual

Rite-Hite EOL Test Fixture Documentation

This is the primary operating guide for technicians and operators using the Rite-Hite End-of-Line (EOL) Test Fixture and EOL Export Software.

It explains:

What the EOL Test Fixture and EOL Export Software do

How to run the normal production workflow

How to use the One Click Test workflow

How to export data and find .csv results

What the operator must still decide manually

For engineering detail, see EOL Test Fixture Architecture & Design.

# System Overview

The station is made up of three connected pieces:

The STM32U575 EOL Test Fixture, which runs the touchscreen UI and the test sequence.

The Rite-Hite Connect Module running the ESP32-S3 test firmware

The Windows EOL Export Software, which uploads DUT test firmware, exports results, and can run the One Click Test workflow (all-in-one test)

The EOL Test Fixture is the source of truth for the test result. It creates the Pass/Fail report row by row while the test runs. The EOL Export Software reads that completed report data and saves it to .csv.

# What Is Tested

The automatic production flow covers the following sequence:

Communication

MCU Reset

Fault Check

Digital Inputs

Analog Inputs

Digital Outputs

Relay Outputs

Buttons

LED Visual Check

## Current Revision Notes

Digital Output 1 and Digital Output 2 are temporary auto-pass report rows.

The fixture automatically drives the button sequence during the Buttons stage.

The technician still makes the final LED decision by pressing LED's GOOD or LED's BAD on the touchscreen.

Failed runs remain exportable.

The EOL Export Software production firmware upload feature is not available yet.

# Before You Start

Before running any test:

- Power the fixture and confirm the touchscreen starts normally.

Verify that the Rite-Hite Connect Module (DUT) or harness is connected correctly.

Verify that the correct test firmware is present on the Rite-Hite Connect Module, or confirm that the programmer USB-C cable “UPLOAD” is available so the EOL Export Software can upload it.

Confirm that the EOL Export Software opens and the exports folder is available to be used.

Review EOL Test Fixture Preflight Checklist if station readiness is in doubt.

# Fixture Workflow

The fixture itself has two entry paths.

## Begin Automatic Test

This is the normal production path.

Use Begin Automatic Test when you want the fixture to run the complete automatic test sequence without the need for software. In this case, the EOL Export Software will just export the data.

## Begin Manual Test

This is the service and debug path.

Use Begin Manual Test only when engineering or maintenance instructs you to use the manual configuration service screen. The EOL Export Software cannot be used with this test mode.

# Standard Production Workflow

This is the normal workflow when the operator starts the test on the EOL Test Fixture touchscreen and exports afterward on the EOL Export Software from the PC.

## Step 1: Start The Fixture Run

On the fixture touchscreen:

- Confirm the start screen is visible.

Press Begin Automatic Test.

## Step 2: Let The Automatic Stages Run

The fixture then runs:

Communication

MCU Reset

Fault Check

Digital Inputs

Analog Inputs

Outputs

Buttons

During these stages:

Observe the screen.

Do not disconnect the DUT.

Do not disconnect the fixture USB or programmer.

Do not restart the run unless a fault requires a new test attempt after correction.

## Step 3: Make The LED Decision

At the end of the run, the fixture reaches the LED visual review.

At that point:

Look at the LED behavior on the fixture.

Press LED's GOOD if the LED behavior correctly matches the behavior.

Press LED's BAD if the LED behavior incorrectly matches the behavior.

## Step 4: Record The Result

If the fixture reaches the Test Complete Screen, the run passed.

If the fixture reaches the Fault Screen, the run failed.

On a failure, record:

The displayed hex fault code

The failed step name

The expected value

The actual value

## Step 5: Export The Data .csv

In the EOL Export Software:

- Go to Device Tools.

Press Extract Data from Test Fixture.

Save or confirm the data .csv.

Verify that the file appears in File Explorer and it can be opened.

# One Click Test Workflow

One Click Test is the PC-driven workflow that uploads firmware, starts the fixture automatically, tracks the live stage, and exports the .csv automatically at the end through the EOL Export Software.

## What One Click Test Does

When you press Start One Click Test, the app:

- Uploads the Rite-Hite Connect Module test firmware

Waits for the module to boot

Sends Begin Automatic Test to the fixture

Polls the fixture status until the test completes

Waits for the LED visual decision by the technician

Exports the final .csv data file automatically

## Before Starting One Click Test

On the One Click Test page, confirm:

The EOL Test Fixture is detected by plugging in the USB-C cable labeled “EXPORT”.

The programmer is detected by plugging in the USB-C cable labeled “UPLOAD”.

The start button is enabled

## Running One Click Test

- Press Start One Click Test.

Watch the progress rail and the Activity Log.

Follow any technician prompt shown in the Activity Log.

## One Click Test Progress Rail

The One Click Test progress rail follows the real fixture stage:

- Upload Firmware

Comms

Digital Inputs

Analog Inputs

Digital Outputs

Relay Outputs

Buttons

LEDs

Extract Data

Important behavior:

The current stage is shown in red.

Completed stages turn green.

The EOL Export Software updates when the EOL Test Fixture changes stage.

The EOL Export Software is not using a fake timer or fixed delay to move the progress bar.

## Technician Action During One Click Test

The EOL Export Software still requires the technician to make the final LED decision on the fixture.

When the fixture reaches the LED stage:

- The Activity Log shows a technician action prompt.

Go to the fixture touchscreen.

Press LED's GOOD or LED's BAD.

## What One Click Test Locks While Running

While One Click Test is active, the EOL Export Software blocks:

Manual data extract

Manual firmware upload

Starting another One Click Test run

This prevents the active run from being interrupted or exported early.

# EOL Export Software Pages

## File Explorer

Use File Explorer to:

Confirm that a .csv was created for data export

Search exported files by filename

Filter by result outcome

Open the exports folder

Change the exports folder

Reset to the default exports folder

### Default Export Location

By default, exports are stored in the EOL Export Software’s exports folder. The export location can be changed in the app and is stored locally on the PC.

### Data Format (.csv)

Each exported .csv data file contains these columns:

Date and Time

Test name

Expected Result

Actual result

Pass/Fail

File names use this pattern:

YYYY-MM-DD_HH-MM-SS_<outcome>.csv

## Device Tools

Use Device Tools for manual PC-side actions.

### Available Actions

Extract Data from Test Fixture

Upload Test Firmware

Upload Production Firmware

### Current Limitation

Upload Production Firmware currently opens a coming soon message. It is not an active production operation yet due to the development cycle.

### Last Successful Operation

The Device Tools page also shows the last successful export or last successful test-firmware upload recorded on that PC.

## One Click Test

Use this page when you want the EOL Export Software to run the full workflow.

It includes:

A connection-readiness card

A start button

The live progress rail

## Activity Log

Use the Activity Log to see:

Upload progress

Start and stop messages

One Click Test stage changes

LED technician prompts

Export completion

Error messages

## Settings

The Settings dialog currently exposes:

Language

Confirm before extract

Auto refresh on tab switch

Reduced motion

Dark mode preference

The export folder path is managed from File Explorer and stored locally on the PC.

The current supported interface languages are:

English

Spanish

Hmong

# Keyboard Shortcuts

The current EOL Export Software includes these useful shortcuts:

F5: Refresh file list

Ctrl+O: Open exports folder

Ctrl+E: Extract data from the fixture

Ctrl+U: Upload test firmware

Ctrl+L: Show or hide the Activity Log

Ctrl+B: Collapse or expand the sidebar

Esc: Exit fullscreen or maximized mode

# What To Do On Pass

If the run passes:

## Standard Production Workflow

- Confirm the EOL Test Fixture shows the Test Complete Screen on the touchscreen.

Export the data using the EOL Export Software and confirm it can be viewed in the File Explorer.

Record the unit as passed according to plant procedure.

## One Click Test Workflow

- Confirm the EOL Export Software shows that One Click Test was completed successfully.

Confirm the data exported as a .csv in the EOL Export Software in File Explorer.

Record the unit as passed according to plant procedure.

# What To Do On Fail

If the run fails:

- Record the fault code and failed step from the Fault Screen on the EOL Test Fixture.

Export the failed run using the EOL Export Software.

Use EOL Test Fixture Troubleshooting Guide.

Use EOL Test Fixture Error Code Reference for exact fault meaning.

Correct the issue before restarting the test.

# Operator Rules

Use Begin Automatic Test for normal production runs.

Use Begin Manual Test only for service, debug, or engineering work.

Do not bypass communication, reset, or fault-check failures.

Do not mark a unit as passed unless the EOL Test Fixture completes successfully and the final LED decision is GOOD.

Do not remove a failed Rite Hite Connect Module unit until the required failure information and data export .csv have been captured.

Do not assume the production firmware upload button is active; it is not implemented yet.

# Related Documents

EOL Test Fixture Preflight Checklist

EOL Test Fixture Screen Guide

EOL Test Fixture Test Procedure

EOL Test Fixture Troubleshooting Guide

EOL Test Fixture Error Code Reference