# EOL Test Fixture Project Knowledge Base

Rite-Hite EOL Test Fixture Documentation

This document is the engineering reference for the current Rite-Hite EOL test fixture implementation.

It is intended to answer:

How the fixture actually works today

Which module owns each part of the workflow

What protocol messages and timings are in use

How the desktop app, STM32 firmware, TouchGFX UI, and ESP32 test firmware fit together

This file is intentionally based on current code behavior, not planned future behavior.

# 1. Project Summary

| Item | Value |
| --- | --- |
| Project | EOL_TestFixture_Final |
| Main controller | STM32U575 |
| UI | TouchGFX on STM32 |
| Companion target | ESP32-S3 Rite-Hite Connect test firmware |
| Desktop app | PySide6 / PyInstaller Windows app |
| Primary desktop workflows | Manual export, test-firmware upload, One Click |

# 2. System Responsibilities

## STM32 Fixture

The STM32 is the owner of the test flow. It:

Runs the TouchGFX touchscreen UI

Drives fixture GPIO and analog outputs

Executes all test engines

Builds the pass/fail report

Exposes the export UART service

Accepts START_AUTO from the desktop app

## ESP32 Test Firmware

The ESP32 responds to the fixture protocol. It:

Replies to comm and fault-check tokens

Measures digital input and analog responses

Handles relay and output acknowledgments

Reports button-state and LED-control acknowledgments

## Desktop App

The desktop app is a station tool. It:

Uploads the Rite-Hite Connect test firmware

Queries the fixture export service

Exports report rows to CSV

Tracks One Click progress live

Shows the Activity Log and file history

# 3. High-Level Workflows

# Fixture-Only Workflow

1. Operator presses Begin AUTO on the fixture.

2. Fixture runs the automatic flow.

3. Technician makes the final LED decision on the touchscreen.

4. Desktop app exports the completed run from Device Tools.

# One Click Workflow

1. Desktop app uploads the Rite-Hite Connect test firmware.

2. Desktop app waits 5 seconds for boot.

3. Desktop app sends START_AUTO to the fixture.

4. Fixture runs the same real automatic flow used by the touchscreen.

5. Desktop app polls the fixture status every 0.5 seconds.

6. Technician still makes the final LED decision on the touchscreen.

7. Desktop app exports the completed run automatically.

Important detail:

One Click is not a separate fake test path.

It reuses the real fixture engines and TouchGFX screen flow.

# 4. Touchscreen Navigation Flow

The fixture screens are:

Start_Screen

Test_GPIO_Screen

Comm_Reset_Screen

Inputs_Screen

Outputs_Screen

Buttons_LEDs_Screen

Fault_Screen

Test_Complete_Screen

## Normal Production Flow

Start_Screen -> Comm_Reset_Screen -> Inputs_Screen -> Outputs_Screen -> Buttons_LEDs_Screen -> Test_Complete_Screen

Any test failure redirects to:

Fault_Screen

## Manual Service Flow

Start_Screen -> Test_GPIO_Screen

# 5. Auto-Test Ownership

The auto-test helper only manages start intent and phase tracking.

## Auto-Test State

The STM32 tracks:

whether an auto test start was requested

whether auto mode is active

the current auto phase

## Auto Phases Reported To The Desktop App

COMMS

DIGITAL_INPUTS

ANALOG_INPUTS

DIGITAL_OUTPUTS

RELAY_OUTPUTS

BUTTONS

LEDS

The phase is surfaced through the export UART status response so the desktop app can drive the One Click progress rail from the real fixture state.

# 6. Stage-By-Stage Firmware Behavior

# Communication Test

Purpose:

Verify basic serial communication

Verify the reset path

Verify the fault-check path

## Messages

STM32 sends PING\n

ESP32 replies PONG

STM32 sends READY\n

ESP32 replies OK

## Timing

Receive timeout per attempt: 3000 ms

Delay between PING attempts: 3000 ms

Maximum retries: 3

MCU reset pulse: 1000 ms

MCU reset recovery: 1000 ms

Fault-check pulse: 1000 ms

Fault-check recovery: 1000 ms

Red-fault display before Fault_Screen: 3000 ms

## Fault Codes

0x0101: no response to PING

0x0102: bad PING reply

0x0201: no reconnect after reset

0x0202: bad reply after reset

0x0203: reset pulse verify failure

0x0301: no OK after fault pulse

0x0302: bad OK reply after fault pulse

0x0303: fault pulse verify failure

## UI Tie-In

Comm_Reset_ScreenPresenter::activate() starts the comm test through the model.

On success, the flow advances to Inputs_Screen.

On failure, fault data is populated and Fault_Screen is shown.

# Digital Input Test

Purpose:

Validate 12 digital channels by driving a high-to-low transition and checking the ESP32 response.

## Command Pattern

For each channel 1 to 12:

STM32 sends DIxTEST

STM32 drives the corresponding output HIGH, then LOW

ESP32 replies with DIxOK or DIxFAIL

## Current Behavior

Each channel allows up to 2 attempts.

The expected logical result recorded in the report is High then Low.

## Fault Codes

0x0401: no reply

0x0402: fail token returned

0x0403: invalid reply

# Analog Input Test

Purpose:

Validate 4 analog channels at three levels each.

## Channels

Channels 1 and 2: current loop

Channels 3 and 4: voltage

## DAC Slider Setpoints

| Channel Type | Low | Mid | High |
| --- | --- | --- | --- |
| Current loop channels | 62 | 196 | 330 |
| Voltage channels | 0 | 165 | 330 |

## Engineering Targets

| Channel Type | Low | Mid | High |
| --- | --- | --- | --- |
| Current loop channels | 4 mA | 12 mA | 20 mA |
| Voltage channels | 0 V | 5 V | 10 V |

## Tolerances

Current loop: plus or minus 2 mA

Voltage: plus or minus 1 V

## Request Pattern

The STM32 requests individual points with:

AI1_LOW_REQ

AI1_MID_REQ

AI1_HIGH_REQ

and equivalent tokens for channels 2 through 4.

The ESP32 replies with:

OK_AI<channel>_<level>:<volts>

The analog reply value is interpreted as the measured ADC-side voltage, then converted to engineering units on the STM32 side.

## Conversion Logic

Current loop channels use adcVolts * (1000 / 150)

Voltage channels use adcVolts * 5.1

## Fault Codes

0x0501: no analog reply

0x0502: invalid analog reply

0x0503: measured value out of tolerance

# Output Test

Purpose:

Cover digital outputs and relays.

## Current Revision Behavior

Digital Output 1 and Digital Output 2 are temporary auto-pass report rows. The fixture records them as:

Digital Output 1 / Temporary skip / Auto-pass (no DO board)

Digital Output 2 / Temporary skip / Auto-pass (no DO board)

## Live Relay Validation

The relay portion is active and drives:

K1_ON

K1_OFF

K2_ON

K2_OFF

Expected acknowledgments:

OK_K1_ON

OK_K1_OFF

OK_K2_ON

OK_K2_OFF

Readback validation uses:

K1 NO and NC

K2 NO and NC

## Fault Codes

0x0601: no relay acknowledgment

0x0602: invalid relay acknowledgment

0x0603: relay state mismatch

# Buttons And LEDs Test

Purpose:

Validate button-state detection and LED control.

## Important Current Behavior

The current firmware automatically drives the button sequence from the fixture side. The technician does not manually press the hardware buttons in the normal automatic run.

The only required human action is the final LED decision:

LED's GOOD

LED's BAD

## Button Order

USR_0

PNL_0

PNL_1

PNL_2

PNL_3

## Core Tokens

BTN_BEGIN

LED_BEGIN

LED_ROTATE_250

LED_STOP_ALL_ON

button press and release tokens such as BTN_USR0_PRESS

positive acknowledgments such as OK_BTN_USR0_PRESS

negative acknowledgments such as BTN_USR0_PRESS_HIGH

## Timing

Init timeout: 3000 ms

Button poll timeout: 100 ms

Press action timeout: 1500 ms

Release action timeout: 2000 ms

Assist pre-press delay: 100 ms

Press settle: 50 ms

Release settle: 750 ms

## Visual-Wait Behavior

When the button sequence completes:

The model changes the auto phase to LEDS

The export status service reports WAITING_VISUAL|LEDS

The desktop app logs a technician prompt

The fixture waits until ButtonLedTest_SubmitVisualResult() is called from the touchscreen GOOD or BAD button

## Fault Codes

0x0701: no acknowledgment

0x0702: invalid acknowledgment

0x0703: operator rejected LED visual check

0x0704: timeout

0x0705: state mismatch

# 7. TouchGFX Model Integration

The TouchGFX model drives the test engines and screen transitions.

Key behavior:

Model::tick() services ExportUart_Tick()

Model::tick() consumes pending auto-test start requests

triggerAutoTestScreenTransition() jumps directly to Comm_Reset_Screen

Each presenter starts its own stage through the model

Output-test relay progress is used to switch the auto phase from DIGITAL_OUTPUTS to RELAY_OUTPUTS

Button/LED progress is used to surface the final visual-wait state

# 8. Report Model

The fixture stores a run report internally.

## Report Characteristics

Maximum rows: 48

Row fields:

pass/fail

test name

expected

actual

## Report States

EMPTY

RUNNING

READY

EXPORTED

## Run Completion

Pass finalization increments the run sequence and marks the report READY

Fail finalization also produces a completed exportable report

Successful export marks the run EXPORTED

Important detail:

Failed runs remain exportable after the fixture reaches Fault_Screen

# 9. Export UART Service

The export service runs on the STM32 and is used by the desktop app.

## Supported Commands

EXPORT_STATUS

EXPORT_RUN|<sequence>

START_AUTO

## Common Replies

STATUS|EMPTY

STATUS|RUNNING|<phase>

STATUS|WAITING_VISUAL|LEDS

STATUS|READY|PASS|<runSequence>|<rowCount>

STATUS|EXPORTED|FAIL|<runSequence>|<rowCount>

AUTO_STARTED

AUTO_ERROR|ALREADY_RUNNING

EXPORT_BEGIN|...

ROW|...

EXPORT_END|...

## Export Errors

EXPORT_ERROR|NOT_READY

EXPORT_ERROR|BAD_SEQ

EXPORT_ERROR|ALREADY_EXPORTED

# 10. Desktop App Behavior

The desktop app has three primary pages:

File Explorer

Device Tools

One Click Test

It also includes:

Activity Log drawer

Settings dialog

Last successful operation summary

## Persistent Local Files

The app stores:

exports folder for CSV output

settings.json

last_successful_operation.json

## App Settings

dark_mode

language

font_size

confirm_before_extract

auto_refresh_on_tab_switch

reduced_motion

optional custom exports_dir

## CSV Output

CSV columns are:

Date and Time

Test name

Expected Result

Actual result

Pass/Fail

## One Click Worker

The One Click worker currently uses:

Poll interval: 0.5 seconds

Poll timeout: 300 seconds

Rite-Hite Connect boot wait: 5 seconds

The worker flow is:

1. Upload test firmware

2. Wait for boot

3. Start auto test

4. Poll fixture status

5. Wait for LED decision

6. Export report

7. Write CSV

## One Click UI Behavior

The progress rail follows the real fixture phase.

Current step is red.

Completed steps turn green.

Manual extract and manual test-firmware upload are disabled during a One Click run.

Status and prompt detail is written to the Activity Log.

# 11. Desktop App Limitations

Production firmware upload is present in the UI but not implemented. The button shows a Coming Soon dialog.

One Click depends on both the fixture serial path and the programmer path being available.

The LED decision remains a fixture-side manual action by design.

# 12. Code-Defined Hardware Map

This section only lists signals that are directly defined in the current codebase.

# STM32 Fixture GPIO Map

## Digital Outputs

| Signal | Pin |
| --- | --- |
| DI1 | PF5 |
| DI2 | PF3 |
| DI3 | PD2 |
| DI4 | PC12 |
| DI5 | PC11 |
| DI6 | PC10 |
| DI7 | PC9 |
| DI8 | PC8 |
| DI9 | PA3 |
| DI10 | PC3 |
| DI11 | PC1 |
| DI12 | PC0 |

## Auxiliary Buttons

| Signal | Pin |
| --- | --- |
| AUX0BTN | PE0 |
| AUX1BTN | PE3 |
| AUX2BTN | PF8 |
| AUX3BTN | PF2 |

## Control Buttons

| Signal | Pin |
| --- | --- |
| USRBTN | PB2 |
| MCUBTN | PB11 |
| FLTBTN | PA8 |

## DAC / Mux Control

| Signal | Pin |
| --- | --- |
| DACA0 | PB3 |
| DACA1 | PC6 |
| DACEN | PD11 |
| DAC_OUT | PA4 |

## Digital Readback Inputs

| Signal | Pin |
| --- | --- |
| K1NO | PB13 |
| K1NC | PD12 |
| K2NO | PB4 |
| K2NC | PB5 |
| DO1 | PB1 |
| DO2 | PC2 |

# ESP32 Test Firmware Pins Used In Code

| Signal | Pin |
| --- | --- |
| Serial1 RX from fixture | GPIO11 |
| Serial1 TX to fixture | GPIO10 |
| I2C SDA | GPIO15 |
| I2C SCL | GPIO16 |
| I/O bus reset | GPIO9 |
| Q3 interrupt | GPIO17 |
| Q4 interrupt | GPIO18 |
| ADS alert | GPIO8 |

# 13. Known Revision Constraints

The outputs screen still contains temporary DO1 and DO2 auto-pass placeholders.

The final production-firmware upload workflow is not implemented in the desktop app.

The exported CSV contains human-readable report rows but not a dedicated raw fault-code column.

# 14. Documentation Maintenance Rule

If any of the following change, update this file in the same change set:

protocol tokens

fault codes

stage timings

GPIO ownership

TouchGFX screen flow

One Click behavior

desktop app export behavior