# EOL Test Fixture Documentation Overview

Rite-Hite EOL Test Fixture Documentation

This folder contains the working documentation set for the Rite-Hite end-of-line test fixture.

The system has three major parts:

An STM32U575-based fixture that runs the test flow and TouchGFX touchscreen UI

An ESP32-S3 test target firmware that responds to the fixture protocol

A Windows PySide6 desktop app that uploads test firmware, exports CSV results, and runs the One Click workflow

This documentation set is written for two audiences:

Technicians and operators who need to run the station correctly and recover from failures

Engineers who need to maintain the firmware, TouchGFX screens, serial protocol, and desktop app

# Start Here

Operators: begin with EOL Test Fixture User Manual

Engineers: begin with EOL Test Fixture Project Knowledge Base

# Operator Documents

EOL Test Fixture User Manual: Primary station operating guide

EOL Test Fixture Preflight Checklist: Station readiness and shift-start checklist

EOL Test Fixture Screen Guide: Touchscreen and desktop-app screen reference

End-of-Line Test Procedure: Formal production and One Click procedures

EOL Test Fixture Troubleshooting Guide: Operator recovery steps and station issues

End-of-Line Error Code Reference: Fault codes, protocol states, and export errors

# Engineering Documents

EOL Test Fixture Project Knowledge Base: Architecture, protocol, hardware map, and stage behavior

Project Firmware And Maintenance Reference: Project-specific firmware and maintenance reference

# Current System Behavior Summary

The fixture touchscreen supports Begin AUTO for the production flow and Begin MANUAL for the service GPIO screen.

The automatic fixture flow runs in this order: Communication, MCU Reset, Fault Check, Digital Inputs, Analog Inputs, Outputs, Buttons, LED Visual Check.

The fixture automatically drives the button sequence during the Buttons stage. The technician does not manually press the hardware buttons during the normal automatic flow.

The final LED decision is still manual. The technician must press LED's GOOD or LED's BAD on the touchscreen.

Failed runs remain exportable after the fixture reaches Fault_Screen.

The desktop app has three working pages: File Explorer, Device Tools, and One Click Test.

One Click uploads the Rite-Hite Connect test firmware, starts the fixture automatic test, tracks live progress, waits for the LED decision, and exports the final CSV automatically.

The One Click progress rail follows the real fixture stage in this order: Upload Firmware, Comms, Digital Inputs, Analog Inputs, Digital Outputs, Relay Outputs, Buttons, LEDs, Extract Data.

Digital Output 1 and Digital Output 2 are still temporary auto-pass rows in the current firmware revision.

The desktop app shows a production firmware upload card, but the production upload action is not available yet.

# Documentation Update Rule

If firmware behavior, protocol tokens, screen flow, desktop app workflow, or station wiring changes, update these markdown files in the same change set. The documents in this folder are intended to match the current software exactly, not a planned future state.