# EOL Test Fixture Error Code Reference

Rite-Hite EOL Test Fixture Documentation

This document is the current reference for:

STM32 fixture fault codes shown on Fault_Screen

Fixture report states used by export and One Click

Export protocol error strings used by the desktop app

Use this with EOL Test Fixture Troubleshooting Guide for recovery steps.

# How To Read The Fault Codes

The fixture uses one shared fault-code namespace.

| Range | Stage |
| --- | --- |
| 0x0000 | No error |
| 0x01xx | Communication |
| 0x02xx | MCU Reset |
| 0x03xx | Fault Check |
| 0x04xx | Digital Inputs |
| 0x05xx | Analog Inputs |
| 0x06xx | Outputs / Relays |
| 0x07xx | Buttons / LEDs |

The fixture displays these codes on Fault_Screen along with:

Failed step name

Expected value or condition

Actual value or condition

# Fixture Fault Codes

| Code | Stage | Meaning | Typical Fault-Screen Context | First Checks |
| --- | --- | --- | --- | --- |
| 0x0000 | All | No error | No fault screen; stage passed | None |
| 0x0101 | Communication | No reply to PING | Communication, expected PONG response, actual No response | Power, UART wiring, test firmware |
| 0x0102 | Communication | Reply received but not PONG | Communication, expected PONG response, actual Invalid reply | Protocol mismatch, wrong firmware |
| 0x0201 | MCU Reset | No reconnect after reset | MCU Reset, expected PONG response, actual No response | Reset path, boot behavior |
| 0x0202 | MCU Reset | Reply after reset was not PONG | MCU Reset, expected PONG response, actual Invalid reply | Post-reset protocol path |
| 0x0203 | MCU Reset | Reset pulse verify failed | MCU Reset, expected reset pulse plus reconnect, actual pulse verify failure | Reset GPIO path |
| 0x0301 | Fault Check | No OK reply after fault pulse | Fault Check, expected OK response, actual No response | Fault line wiring |
| 0x0302 | Fault Check | Reply after fault pulse was not OK | Fault Check, expected OK response, actual Invalid reply | Fault-check protocol path |
| 0x0303 | Fault Check | Fault pulse verify failed | Fault Check, expected fault pulse plus OK, actual pulse verify failure | Fault GPIO path |
| 0x0401 | Digital Inputs | No digital-input reply | Digital Input X, expected High then Low, actual No response | Input wiring, target response |
| 0x0402 | Digital Inputs | Target returned fail token | Digital Input X, expected High then Low, actual Invalid transition | Input state path |
| 0x0403 | Digital Inputs | Invalid digital-input reply | Digital Input X, expected High then Low, actual Invalid reply | Protocol token mismatch |
| 0x0501 | Analog Inputs | No analog reply | Analog Input X - LEVEL, expected sample reply, actual No response | Analog route, target response |
| 0x0502 | Analog Inputs | Invalid analog reply | Analog Input X - LEVEL, expected OK_AI..., actual Invalid reply | Analog protocol token |
| 0x0503 | Analog Inputs | Analog reading out of tolerance | Analog Input X - LEVEL, expected tolerance band, actual measured value | Wiring, scaling, measurement path |
| 0x0601 | Outputs | No relay acknowledgment | Relay K1 ON, Relay K1 OFF, Relay K2 ON, or Relay K2 OFF with No response | Relay command path |
| 0x0602 | Outputs | Invalid relay acknowledgment | Relay step with Invalid reply | Relay protocol token |
| 0x0603 | Outputs | Relay state mismatch | Relay step with expected NO/NC state vs actual state | Relay readback wiring |
| 0x0701 | Buttons / LEDs | No acknowledgment during button or LED stage | Button or LED step with No response | Button/LED interface, target reply |
| 0x0702 | Buttons / LEDs | Invalid acknowledgment during button or LED stage | Button or LED step with Invalid reply | Button/LED protocol token |
| 0x0703 | Buttons / LEDs | Operator rejected LED visual check | LED Visual Check, expected Operator pressed GOOD, actual Operator pressed BAD | LED behavior review |
| 0x0704 | Buttons / LEDs | Button stage timeout | Button step, expected Pressed and released, actual Press timeout or Release timeout | Automatic button-detect path |
| 0x0705 | Buttons / LEDs | Button state mismatch | Button step, expected one button state, actual opposite state | Q3 input readback, button wiring |

# Current Revision Notes

0x060x faults apply to relay validation in the current revision. Digital Output 1 and Digital Output 2 are temporary auto-pass rows.

The technician does not manually press the hardware button sequence during the normal automatic test. The fixture drives those button states automatically.

The technician still chooses LED's GOOD or LED's BAD, which is why 0x0703 still exists as a real operator decision.

# Fixture Report States

The fixture report service uses these run states:

| State | Meaning | What The App Does |
| --- | --- | --- |
| EMPTY | No completed run is available | Export is blocked |
| RUNNING | Test is still active | Export is blocked |
| WAITING_VISUAL | Fixture is waiting for the LED visual decision | Export is blocked and One Click prompts the technician |
| READY | Completed run is ready to export | Manual export and One Click export are allowed |
| EXPORTED | Run has already been exported | The desktop app treats the run as already exported |

## One Click Phase Field

When the fixture is in auto mode, the desktop app can also receive the active phase:

COMMS

DIGITAL_INPUTS

ANALOG_INPUTS

DIGITAL_OUTPUTS

RELAY_OUTPUTS

BUTTONS

LEDS

Examples:

STATUS|RUNNING|COMMS

STATUS|WAITING_VISUAL|LEDS

STATUS|READY|PASS|12|18

# Export Protocol Errors

These are not fixture Fault_Screen codes. They are serial protocol errors used between the desktop app and the export service.

| Message | Meaning | Typical Cause |
| --- | --- | --- |
| EXPORT_ERROR¦NOT_READY | Run is not ready to export | Test still running or waiting for LED decision |
| EXPORT_ERROR¦BAD_SEQ | Requested run sequence does not match | App requested the wrong report sequence |
| EXPORT_ERROR¦ALREADY_EXPORTED | Run already exported | A CSV was already pulled for that run |

# Auto-Test Start Errors

These are also protocol-level messages, not Fault_Screen codes.

| Message | Meaning | Typical Cause |
| --- | --- | --- |
| AUTO_STARTED | Fixture accepted START_AUTO | Normal One Click start |
| AUTO_ERROR¦ALREADY_RUNNING | Fixture rejected START_AUTO because an auto run is active | A previous auto run is still running |

# How Codes Flow Through The System

1. A test engine detects a failure and sets an EOL_ERR_* code.

2. The active TouchGFX screen fills the shared fault information fields.

3. The fixture transitions to Fault_Screen.

4. The fixture report stores the human-readable failed row.

5. The desktop app exports the report rows to CSV.

Important detail:

The exported CSV is based on report rows such as test name, expected, actual, and pass/fail.

The CSV does not have a dedicated fault-code column.

The hex fault code is primarily shown live on the fixture screen.

# Related Documents

EOL Test Fixture Troubleshooting Guide

EOL Test Fixture User Manual

End-of-Line Test Procedure